Network administration – the final test

Multiple choice questions

- Different computers are connected to a LAN by a cable and

- Telephone lines

- Special wires

- Interface card

- Modem

|  | - 2. How long is an IPv6 address? |
| --- | --- |
|  | - 32 bits. - 48 bits. - 64 bits. - 128 bits. |

- What flavor of Network Address Translation can be used to have one IP address allow many users to connect to the global Internet?

- NAT

- Static

- Dynamic

- PAT

- What command is used to create a backup configuration?

- copy running backup

- copy running-config startup-config

- config mem

- wr mem

- Which of the following is the valid host range for the subnet on which the IP address 192.168.168.188 255.255.255.192 resides?

- 192.168.168.129-190

- 192.168.168.129-191

- 192.168.168.128-190

- 192.168.168.128-192

- To back up an IOS, what command will you use?

- backup IOS disk

- copy ios tftp

- copy tftp flash

- copy flash tftp

- Where is a hub specified in the OSI model?

- Session layer

- Physical layer

- Data Link layer

- Application layer

- What is the function of a switch in a Cisco network?

- To connect multiple LANs together

- To connect a LAN to a WAN

- To provide power to network devices

- To manage network security

- Which protocol is used to send a destination network unknown message back to originating hosts?

- TCP

- ARP

- ICMP

- BootP

- What is the function of a VLAN in a Cisco network?

- To divide a physical network into logical networks

- To connect multiple LANs together

- To provide power to network devices

- To manage network security

- What is the command to configure an IP address on a Cisco router interface?

- ip address

- interface ip address

- ip interface

- address ip

- What is the purpose of a VLAN?

- To provide wireless access to the network

- To isolate network traffic between devices

- To connect multiple networks together

- To assign unique IP addresses to devices

- Which of the following is the most common central device used today to connect computers to a network?

- Hub

- Switch

- SOHO router

- VPN router

- Which routing protocol uses a metric that is based on bandwidth and delay?

- RIP

- OSPF

- EIGRP

- BGP

- Which of the following is used to isolate a group of computers within an organization?

- WLAN

- WAN

- VLAN

- Internet

- Which type of cable is used to connect a PC to a switch?

- Crossover cable

- Straight-through cable

- Rollover cable

- Fiber optic cable

- Which protocol is used to automatically assign IP addresses to devices on a network?

- DNS

- SMTP

- DHCP

- FTP

- Which command is used to configure a static route in Cisco routers?

- ip route

- route add

- add route

- configure route

- Which model is used to describe how data communication occurs between hosts?

- Server-centric model

- Workgroup model

- Peer-to-peer model

- OSI reference model

Short answers

- The application layer in the OSI model covers HTTP, FTP, and RDC.

- The session layer of the OSI model is used to create a connection so that a host can transfer files.

- The data link layer of the OSI model includes VLANs.

- The UDP protocol is used as the transport protocol for a video application.

- The physical layer in the OSI model includes the cable and network adapters.

- The transport layer in the OSI model is included in the TCP/IP model.

- A layer 3 switch is similar to a router.

- A host is a single computer or device that connects to a TCP/IP network.

- If you want to connect a computer directly to another computer without using a switch, you should use a crossover cable.

- WPA2 is the most secure encryption used in wireless networks.

- SSID is used to identify a wireless network.

- For IPv4, Class D is used for multicasting.

- 254 is the maximum number of hosts you can use for each subnet that belongs to a class C address.

- Multicast address is used to send a single set of packets to multiple hosts.

- Default gateway defines the nearest router for a host.

- DNS server is used to translate host names to IP addresses.

- NAT is used to remap private addresses to public addresses and back.

- Subnet is used to describe a smaller network that is part of a larger network.

- There are 128 bits in an IPv6 address.

- 2001:4212:0000:34DB:0000:0000:0000:4231 is a MAC address.

- A dynamic address is assigned by a DHCP server.

True / False

- IPv4 address has 48 bits – False

- A router is a layer 2 device that forwards data packets based on MAC addresses – False

- In a VLAN, all devices share the same broadcast domain – True

- ipconfig command displays the IP address, subnet mask, and default gateway of the system you are currently on – True

- A subnet mask of 255.255.255.0 means that the network can have up to 254 hosts – True

- The switch operates at the data link layer of the OSI model – True

- DHCP protocol automatically configures IP configuration for a client – True

- Network Address Translation (NAT) allows multiple devices to share a single public IP address – True

- DNS allows a user to connect to a corporate network using the internet – False

- RIP routing protocol is the most popular distance-vector routing algorithm used to determine the best routes within a network – True

- OSPF routing protocol is the most popular link-state protocol used within a large organization – True

- When a packet traverses a network, each router is a hop – True

- A switch is used to send packets from one network to another network – False

- The function of a router is to provide IP subnet masks for hosts – False

- If a router cannot determine the next hop for a packet, the router will forward the packet to the default route – True

- In Star physical network topology each computer is connected to a central point – True

- Extranet is a private network that allows members of an organization to exchange data – False

- To protect a network when it is connected to the Internet, you should use a firewall – True

- HTTP protocol can be used to encrypt packets on the Internet – False

- A network device that associates a Media Access Control (MAC) address with a port is a Router – False

Correspondence questions

- Question 1

| IPv4 address | A unique numerical identifier of a device in a computer network consisting of 32 bits. |
| --- | --- |
| MAC address | unique combination of numbers and letters 48 characters long |
| IPv6 address | A unique numerical identifier of a device in a computer network consisting of 128 bits. |

- Question 2

| Mesh topology | Every computer connects to every other computer; no central connecting device is needed |
| --- | --- |
| Star topology | Each computer is individually wired to a central connecting device (hub, switch or SOHO router) with twisted-pair cabling |
| Ring topology | In a LAN environment, each computer is connected to the network using a closed loop |

- Question 3

| File server | A computer responsible for the storage and management of data files |
| --- | --- |
| Print server | Software or device that allows a group of wired and wireless network users to share a printer in a home or office |
| Database server | A networked computer on a network dedicated to database storage and data retrieval from the database |

- Question 4

| Physical layer | The physical and electrical medium for data transfer |
| --- | --- |
| Datalink layer | This layer establishes, maintains, and decides how transfer is accomplished over the physical layer |
| Network layer | This layer is dedicated to routing and switching information to different networks, LANs |

- Question 5

| Transport layer | This layer ensures error-free transmission between hosts through logical addressing |
| --- | --- |
| Session layer | This layer governs the establishment, termination, and synchronization of sessions within the OS over the network and between hosts |
| Presentation layer | This layer translates the data format from sender to receiver in the various OSes that may be used |
| Application layer | This layer is where message creation—and, therefore packet creation—begins |

- Question 6

| Straight through cable | Used to connect a computer to a central connecting device like a switch |
| --- | --- |
| Crossover cable | Used to connect like devices to each other, for example, a computer to another computer, or a switch to another switch |
| STP cable | Has an aluminum shield inside the plastic jacket that surrounds the pairs of wires |
| Fiber optic cable | Transmits light (photons) instead of electricity, and this light is transmitted over glass or plastic |

- Question 7

| DHCP | An application protocol that allows network devices to automatically obtain an IP address and other parameters |
| --- | --- |
| NAT | A mechanism in TCP / IP networks that allows you to convert the IP addresses of transit packets |
| PAT | Port based address translation technology |

- Question 8

| IPSec | A protocol within the TCP/IP suite that encrypts and authenticates IP packets |
| --- | --- |
| DNS | A worldwide service that resolves host names to IP addresses |
| WINS | A service that resolves NetBIOS names to IP addresses |

- Question 9

| Dynamic routing | is implemented by dynamically configuring routing tables |
| --- | --- |
| Static routing | is one that has been manually configured |
| Routing | is the process of moving data across networks or internetworks between hosts or between routers |

- Question 10

| RIP | A dynamic protocol that uses distance-vector routing algorithms to decipher which route to send data packets |
| --- | --- |
| OSPF | A link-state protocol that monitors the network for routers that have a change in their link state |
| BGP | A core routing protocol that bases routing decisions on the network path and rules |

- Question 11

| LAN | is a computer network that usually covers a relatively small area or group of buildings |
| --- | --- |
| WAN | is a global computer network. Simply put, it's the internet. |
| X.25 | communications protocol was one of the first implementations of packet switching, and it is still in use today |

- Question 12

| DSL | is a family of technologies that provides data transmissions over local telephone networks |
| --- | --- |
| FDDI | is a standard for transmitting data on optical fiber cables at a rate of around 100 Mbps |
| VLAN | a group of hosts with a common set of requirements that interact as if they were connected to a broadcast domain regardless of their physical location |

- Question 13

| VPN | technologies that provide one or more network connections over another network, such as the Internet |
| --- | --- |
| SSH | application-level network protocol that allows remote control of the operating system and tunneling of TCP connections |
| Telnet | this is a means of communication that establishes a transport connection between terminal devices and clients |

- Question 14

| NSLookup | displays information about DNS names and their corresponding IP addresses, and it can be used to diagnose DNS servers |
| --- | --- |
| NetStat | is used to display active TCP (or UDP) connections |
| NbtStat | displays NetBIOS over TCP/IP statistics for local and remote computers |

- Question 15

| Ping | A command for checking the integrity and quality of connections in networks based on TCP/IP |
| --- | --- |
| Tracert | command line utility that can be used to trace the path that an Internet Protocol (IP) packet takes to its destination |
| Ipconfig | one of the most frequently used network utilities in Windows, because it allows you to quickly and most importantly conveniently view the network adapter settings in Windows |

- Question 16

| Network layer | layer of the OSI model responsible for routing data between different networks |
| --- | --- |
| FTP | protocol used to transfer files between devices over a network |
| Transport layer | layer of the OSI model is responsible for establishing a reliable data transmission between applications |

- Question 17

| default gateway | used to forward data packets between different networks |
| --- | --- |
| hub | a layer 1 device that broadcasts all data packets to all connected devices |
| switch | a layer 2 device that selectively forwards data packets to their intended recipients |

- Question 18

| SSH | protocol used to securely access network devices remotely |
| --- | --- |
| loopback interface | used to provide a reliable local address for the device, for testing and management purposes |
| subnet mask | used to determine the network and host portions of an IP address |

- Question 19

| OSPF | routing protocol uses a link-state database to calculate the best path between networks |
| --- | --- |
| VLAN | used to segment a network into smaller broadcast domains for security, scalability, and management purposes |
| ARP | used to store the mappings between IP addresses and MAC addresses for faster data forwarding |

- Question 20

| session layer | layer of the OSI model responsible for establishing sessions between applications |
| --- | --- |
| subnet | used to divide a network into smaller logical segments for better traffic management and security |
| RIP | protocol used to manage the routing table of a Cisco device |