# 14 Cisco Router and Switch Basics

# - Lab Exercise

In this lab you will complete a basic configuration on a switch, verify Cisco Discovery Protocol CDP and analyse the effects of interface speed and duplex configuration.

# Lab Topology

Load the Startup Configurations

Open the ’14 Cisco Router and Switch Basics.pkt’ file in Packet Tracer to load the lab.

# Cisco Router and Switch Initial Configuration

- Configure Router 1 with the hostname ‘R1’

- Configure Router 2 with the hostname ‘R2’

- Configure Switch 1 with the hostname ‘SW1’

- Configure the IP address on R1 according to the topology diagram

- Configure the IP address on R2 according to the topology diagram

- Give SW1 the management IP address 10.10.10.10/24

- The switch should have connectivity to other IP subnets via R2

- Verify the switch can ping its default gateway

- Enter suitable descriptions on the interfaces connecting the devices

- On SW1, verify that speed and duplex are automatically negotiated to 100 Mbps full duplex on the link to R1

- Manually configure full duplex and FastEthernet speed on the link to R2

- What version of IOS is the switch running?

# CDP Configuration

- Verify the directly attached Cisco neighbors using Cisco Discovery Protocol

- Prevent R1 from discovering information about Switch 1 via CDP

- Flush the CDP cache on R1 by entering the ‘no cdp run’ then ‘cdp run’ commands in global configuration mode

- Verify that R1 cannot see SW1 via CDP

# Switch Troubleshooting

- Verify the status of the switch port connected to R2 with the show ip interface brief command. It should show status and protocol up/up.

- Shut down the interface connected to R2 and issue a show ip interface brief command again. The status and protocol should show administratively down/down.

- Bring the interface up again. Verify the speed and duplex setting.

- Set the duplex to half on Switch 1. Leave the settings as they are on R2.

- Verify the state of the interface.

- Set the duplex back to full duplex.

- Set the speed to 10 Mbps.

- Check if the interface is still operational.

- Check if the interface is operational on R2. What is the status of the interface?