# 19-1 IGP Interior Gateway Protocol Fundamentals Configuration – Lab Exercise

In this lab you will configure the RIPv2 and EIGRP routing protocols. IP addresses are already configured on the routers.

# Lab Topology

Load the Startup Configurations

Open the ‘19-1 IGP Fundamentals Configuration.pkt’ file in Packet Tracer to load the lab.

# RIP Configuration

- Enable RIPv2 on every router. Ensure all networks except 203.0.113.0/24 are advertised. Do not perform any summarisation.

- Verify all networks are in the router’s routing tables.

- Verify that routing is working by checking that PC1 has connectivity to PC3.

- Ensure that all routers have a route to the 203.0.113.0/24 network. Internal routes must not advertised to the Service Provider at 203.0.113.2.

- Verify that all routers have a path to the 203.0.113.0/24 network.

- Configure a default static route on R4 to the Internet via the service provider at 203.0.113.2

- Ensure that all other routers learn via RIP how to reach the Internet.

- Verify all routers have a route to the Internet.

# EIGRP Configuration

- Enable EIGRP AS 100 on every router. Ensure all networks except 203.0.113.0/24 are advertised in EIGRP.

- Verify the routers have formed adjacencies with each other.

- Which routing protocol (RIP or EIGRP) do you expect routes to the

10.x.x.x networks to be learned from in the routing tables?

- Do you expect to see any routes from the other routing protocol in the routing tables?

- View the routing tables to verify your answers.